# ECEN 611 Homework 2: Sinusoidal

Shuxuan Chen | UIN: 132006082 | Fall 2024

Last Updated: Sep 22

Table of Contents

Problem 1 
    (a) Winding Function Diagrams for Windings A, B, and C    
        General Settings 
        Counting Functions for Windings A, B, and C    
        Winding Functions for Windings A, B, and C    
        Plot Counting Functions and Winding Functions A, B, and C    
    (b) MMF of Windings A, B, and C Supplied by Three Phase Sinusoidal Currents 
        Three Phase Sinusoidal Currents 
        Three Phase Currents at Angle of Interest 
        Three Phase MMFs at Angles of Interest 
    (c) Total MMF at Angles of Interest 
Problem 2 
    (d) Fourier Series Analysis 
        Winding Functions Represented by Fourier Series 
        Sinusoidal Currents Represented by Fourier Series 
        Phase MMF Represented by Fourier Series at Angles of Interest 
        Total MMF Represented by Fourier Series at Angles of Interest 
        Table

clearvars

clc

Numeric computation is always recommended when it can solve the problem with acceptable precision.

# Problem 1

## (a) Winding Function Diagrams for Windings A, B, and C

Draw the winding functions for windings A, B, and C on the same sheet of paper.

### General Settings

% Define the spatial angle from 0 to 360 degrees

T = 2*pi;

T_degree = rad2deg(T)

T_degree = 360

numOfPoints = 5000; % This affects precision of Fourier analysis

theta = linspace(0, T, numOfPoints);

theta_degrees = linspace(0, T_degree, numOfPoints)

theta_degrees = 1×5000

0    0.0720    0.1440    0.2160    0.2881    0.3601    0.4321 ⋯

### Counting Functions for Windings A, B, and C

% Initialize counting function to zero arrays

nA = zeros(size(theta));

nB = zeros(size(theta));

nC = zeros(size(theta));

% Define nA

nA(mod(theta, T) >= 0 & mod(theta, T) < T/2) = 1;

nA(mod(theta, T) >= T/2 & mod(theta, T) <= T) = 0;

Explanation:

- mod(theta, T): This ensures that the function repeats every T. By applying mod(theta, T), you're effectively folding the waveform to repeat from T to 2T.

- First Condition: For the interval 0≤θ≤T/2, the function returns 1.

- Second Condition: For the interval T/2<θ≤T, the function returns nA_pi - 1.

% Define nB

% nB(theta >= 0 & theta < 120) = 0;

% nB(theta >= 120 & theta < 300) = 1;

% nB(theta >= 300 & theta <= 360) = 0;

%

% % Define nC

% nC(theta >= 0 & theta < 60) = 1;

% nC(theta >= 60 & theta < 240) = 0;

% nC(theta >= 240 & theta <= 360) = 1;

% Define phase shift for iB and iC

phase_shift = 2*pi/3;

% Define iB as phase-shifted iA

nB = circshift(nA, [0, round(phase_shift/(2*pi) * length(theta))]);

% Define iC as phase-shifted iA by 2*phase_shift

nC = circshift(nA, [0, round(2*phase_shift/(2*pi) * length(theta))]);

### Winding Functions for Windings A, B, and C

% Define NA, NB, and NC

NA = nA - mean(nA);

NB = nB - mean(nB);

NC = nC - mean(nC);

### Plot Counting Functions and Winding Functions A, B, and C

figure

tiledlayout(3,1)

titleHandle = sgtitle("Counting Function and Winding Function");

titleHandle.FontSize = 16;

% First tile: nA and NA

nexttile;

plot(theta_degrees, nA, 'LineWidth', 1.2);

hold on;

plot(theta_degrees, NA, 'LineWidth', 1.2);

hold off;

ylim([-1 2]);

grid on;

xlabel("Spatial Angle (°)");

ylabel("Number of Turns");

subtitle("Phase A Winding");

legend('nA', 'NA');

xticks(0:30:360);

% Second tile: nB and NB

nexttile;

plot(theta_degrees, nB, 'LineWidth', 1.2);

hold on;

plot(theta_degrees, NB, 'LineWidth', 1.2);

hold off;

ylim([-1 2]);

grid on;

xlabel("Spatial Angle (°)");

ylabel("Number of Turns");

subtitle("Phase B Winding");

legend('nB', 'NB');

xticks(0:30:360);

% Third tile: nC and NC

nexttile;

plot(theta_degrees, nC, 'LineWidth', 1.2);

hold on;

plot(theta_degrees, NC, 'LineWidth', 1.2);

hold off;

ylim([-1 2]);

grid on;

xlabel("Spatial Angle (°)");

ylabel("Number of Turns");

subtitle("Phase C Winding");

legend('nC', 'NC');

xticks(0:30:360);

% Counting functions √

% Winding functions √

## (b) MMF of Windings A, B, and C Supplied by Three Phase Sinusoidal Currents

Draw the MMF of each phase if the windings are supplied by three phase sinusoidal currents at ωt = 0°, 30° , 60° , 90° , 120°, ...

### Three Phase Sinusoidal Currents

wt = linspace(0, T, numOfPoints);

wt_degrees = rad2deg(wt);

[iA, iB, iC] = generateSinusoidalCurrents(wt);

% Generate sinusoidal currents for three phases

function [iA, iB, iC] = generateSinusoidalCurrents(wt)

iA = cos(wt);

iB = cos(wt - 2*pi/3);

iC = cos(wt + 2*pi/3);

% iA = sin(wt);

% iB = sin(wt - 2*pi/3);

% iC = sin(wt + 2*pi/3);

end

% Ensure wt_degrees, iA, iB, and iC are defined

figure;

plot(wt_degrees, iA, ...

wt_degrees, iB, ...

wt_degrees, iC, 'LineWidth', 1.2);

legend('iA', 'iB', 'iC');

title('Three-Phase Sinusoidal Currents');

xlabel('Degrees');

ylabel('Current (A)');

grid on;

### Three Phase Currents at Angle of Interest

angleOfInterest = 0:30:330;

% Pre-allocate the index array

indices = zeros(size(angleOfInterest));

% Loop through each angle of interest and find the closest match in wt_degrees

for i = 1:length(angleOfInterest)

[~, idx] = min(abs(wt_degrees - angleOfInterest(i))); % Find closest value

indices(i) = idx; % Store the index of the closest value

end

iA_angleOfInterest = iA(indices);

iB_angleOfInterest = iB(indices);

iC_angleOfInterest = iC(indices);

% Ensure wt_degrees, iA, iB, and iC are defined

figure;

plot(wt_degrees(indices), iA_angleOfInterest, ...

wt_degrees(indices), iB_angleOfInterest, ...

wt_degrees(indices), iC_angleOfInterest, 'LineWidth', 1.2);

legend('iA', 'iB', 'iC');

title('Three-Phase Sinusoidal Currents (at Angle of Interest)');

xlabel('Degrees');

ylabel('Current (A)');

grid on;

### Three Phase MMFs at Angles of Interest

MMFA = NA .* iA_angleOfInterest';

MMFB = NB .* iB_angleOfInterest';

MMFC = NC .* iC_angleOfInterest';

for k = 1:length(angleOfInterest)

figure;

tiledlayout(3, 1);

% Common title with current wt in degrees

titleHandle = sgtitle(sprintf('MMF at ωt = %.0f°', angleOfInterest(k)));

titleHandle.FontSize = 13;

% Plot Phase A MMF

nexttile;

plot(wt_degrees, MMFA(k,:), 'LineWidth', 1.2);

setPlotFormatting('Phase A Winding');

% Plot Phase B MMF

nexttile;

plot(wt_degrees, MMFB(k,:), 'LineWidth', 1.2);

setPlotFormatting('Phase B Winding');

% Plot Phase C MMF

nexttile;

plot(wt_degrees, MMFC(k,:), 'LineWidth', 1.2);

setPlotFormatting('Phase C Winding');

end

function setPlotFormatting(subtitleText)

% Helper function to set formatting for a plot

ylim([-1 1]);

grid on;

xlabel("Spatial Angle (°)");

ylabel("MMF (A·Turns)");

subtitle(subtitleText);

xticks(0:30:360);

end

## (c) Total MMF at Angles of Interest

Find the total MMF at ωt = 0°, 30° , 60° , 90° , 120°, ...

totalMMF = MMFA + MMFB + MMFC;

figure;

tiledlayout(3, 4, 'TileSpacing', 'Compact', 'Padding', 'Compact');

sgtitle('Total MMF at Different ωt Degrees');

titleHandle.FontSize = 13;

for k = 1:length(angleOfInterest)

nexttile;

plot(wt_degrees, totalMMF(k,:), 'LineWidth', 1.2);

ylim([-2 2]);

yticks(-2:0.5:2)

grid on;

xlabel("Spatial Angle (°)");

ylabel("MMF (A·Turns)");

title(sprintf('ωt = %.0f°', angleOfInterest(k)))

end

# Problem 2

## (d) Fourier Series Analysis

### Winding Functions Represented by Fourier Series

% a0 = (1/T)*int(expression, theta, 0, T);

% an = (2/T)*int(expression*cos(n*theta), theta, 0, T);

% bn = (2/T)*int(expression*sin(n*theta), theta, 0, T);

% DC component

NA_a0 = mean(NA)

NA_a0 = -5.1816e-15

% Order of harmonics

orderOfHarmonics = 1:13;

N = 1;

NA_bn_ref = 4./(orderOfHarmonics*pi)*N/2.*sin(orderOfHarmonics*pi/2)

NA_bn_ref = 1×13

0.6366    0.0000   -0.2122   -0.0000    0.1273    0.0000   -0.0909 ⋯

% Initialization

NA_an = zeros(size(orderOfHarmonics));

NA_bn = zeros(size(orderOfHarmonics));

for n = 1:length(orderOfHarmonics)

% Amplitude of even harmonics (cos terms)

NA_an(n) = 2*mean(NA.*cos(n*theta));

% Amplitude of odd harmonics (sin terms)

NA_bn(n) = 2*mean(NA.*sin(n*theta));

end

NA_an

NA_an = 1×13

10-3 ×

0.3999    0.3999    0.3999    0.3999    0.3999    0.3999    0.3999 ⋯

NA_bn

NA_bn = 1×13

0.6365   -0.0000    0.2122   -0.0000    0.1273   -0.0000    0.0909 ⋯

[NB_a0, NB_an, NB_bn] = computeHarmonicsAmplitude(NB, theta, orderOfHarmonics)

NB_a0 = -3.7291e-15

NB_an = 1×13

-0.5515   -0.0005   -0.0001   -0.0005    0.1100    0.0004   -0.0790 ⋯

NB_bn = 1×13

-0.3182   -0.0005    0.2122    0.0005   -0.0637   -0.0000   -0.0454 ⋯

[NC_a0, NC_an, NC_bn] = computeHarmonicsAmplitude(NC, theta, orderOfHarmonics)

NC_a0 = -3.7291e-15

NC_an = 1×13

0.5511    0.0001    0.0009    0.0001   -0.1104    0.0004    0.0786 ⋯

NC_bn = 1×13

-0.3189    0.0002    0.2122   -0.0002   -0.0630   -0.0000   -0.0461 ⋯

% Reconstructed winding function

NA_rec = zeros(size(theta));

NB_rec = zeros(size(theta));

NC_rec = zeros(size(theta));

for n = 1:length(orderOfHarmonics)

NA_rec = NA_rec + NA_an(n).*cos(n*theta) + NA_bn(n).*sin(n*theta);

if n == orderOfHarmonics(end)

NA_rec = NA_rec + NA_a0/2;

end

end

NB_rec = reconstructFromFourierSeries(NB_a0, NB_an, NB_bn, theta, orderOfHarmonics);

NC_rec = reconstructFromFourierSeries(NC_a0, NC_an, NC_bn, theta, orderOfHarmonics);

function result = reconstructFromFourierSeries(a0, an, bn, theta, orderOfHarmonics)

result = zeros(size(theta));

for n = 1:length(orderOfHarmonics)

result = result + an(n).*cos(n*theta) + bn(n).*sin(n*theta);

if n == orderOfHarmonics(end)

result = result + a0/2;

end

end

end

figure

tiledlayout(3,1)

titleHandle = sgtitle("Winding Functions Reconstructed from Fourier Series");

titleHandle.FontSize = 12;

nexttile;

plot(wt_degrees,NA_rec,wt_degrees,NA,"LineWidth",1.2)

subtitle("Phase A Winding");

ylabel("Number of Turns");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

nexttile;

plot(wt_degrees,NB_rec,wt_degrees,NB,"LineWidth",1.2)

subtitle("Phase B Winding");

ylabel("Number of Turns");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

nexttile;

plot(wt_degrees,NC_rec,wt_degrees,NC,"LineWidth",1.2)

subtitle("Phase C Winding");

ylabel("Number of Turns");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

% Compare with original ones

Plot Winding Function Odd Harmonics Spectrum

% numOfPhase = 3;

% N_bn = [];

% for k = 1:length(orderOfHarmonics)

%     N_bn = [N_bn; NA_bn(k) NB_bn(k) NC_bn(k)];

% end

%

% figure;

% bar(N_bn, 'grouped'); % 'grouped' creates the grouped bar chart

%

% % Labeling

% xlabel('Order of Harmonics');

% ylabel('Magnitude');

% title('Odd Harmonics Spectrum of Phase A, B, C Winding Functions');

%

% % Customize the x-tick labels

% % set(gca, 'XTickLabel', {'Group 1', 'Group 2', 'Group 3'});

%

% legend({'Winding A', 'Winding B', 'Winding C'}, 'Location', 'best');

% grid on;

### Sinusoidal Currents Represented by Fourier Series

[iA_a0, iA_an, iA_bn] = computeHarmonicsAmplitude(iA, wt, orderOfHarmonics)

iA_a0 = 2.0000e-04

iA_an = 1×13

1.0002    0.0004    0.0004    0.0004    0.0004    0.0004    0.0004 ⋯

iA_bn = 1×13

10-16 ×

0.2467    0.1715    0.3035    0.2122   -0.1285   -0.3443   -0.1322 ⋯

[iB_a0, iB_an, iB_bn] = computeHarmonicsAmplitude(iB, wt, orderOfHarmonics)

iB_a0 = -1.0000e-04

iB_an = 1×13

-0.5001   -0.0002   -0.0002   -0.0002   -0.0002   -0.0002   -0.0002 ⋯

iB_bn = 1×13

0.8659   -0.0000    0.0000    0.0000    0.0000   -0.0000    0.0000 ⋯

[iC_a0, iC_an, iC_bn] = computeHarmonicsAmplitude(iC, wt, orderOfHarmonics)

iC_a0 = -1.0000e-04

iC_an = 1×13

-0.5001   -0.0002   -0.0002   -0.0002   -0.0002   -0.0002   -0.0002 ⋯

iC_bn = 1×13

-0.8659   -0.0000    0.0000    0.0000    0.0000    0.0000   -0.0000 ⋯

iA_rec = reconstructFromFourierSeries(iA_a0, iA_an, iA_bn, wt, orderOfHarmonics);

iB_rec = reconstructFromFourierSeries(iB_a0, iB_an, iB_bn, wt, orderOfHarmonics);

iC_rec = reconstructFromFourierSeries(iC_a0, iC_an, iC_bn, wt, orderOfHarmonics);

figure

tiledlayout(3,1)

titleHandle = sgtitle("Phase Currents Reconstructed from Fourier Series");

titleHandle.FontSize = 12;

nexttile;

plot(wt_degrees,iA_rec, ...

"DisplayName", "Reconstructed", ...

"LineWidth",1.2, ...

"Marker","o")

hold on

plot(wt_degrees,iA, ...

"DisplayName", "Original", ...

"LineWidth",1.2)

hold off

legend

subtitle("Phase A Current");

ylabel("Current (A)");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

nexttile;

plot(wt_degrees,iB_rec,wt_degrees,iB,"LineWidth",1.2)

subtitle("Phase B Current");

ylabel("Current (A)");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

nexttile;

plot(wt_degrees,iC_rec,wt_degrees,iC,"LineWidth",1.2)

subtitle("Phase C Current");

ylabel("Current (A)");

xlabel("Spatial Angle (°)");

xticks(0:30:360)

ylim([-1 1])

grid

### Phase MMF Represented by Fourier Series at Angles of Interest

iA_rec_angleOfInterest = iA_rec(indices);

iB_rec_angleOfInterest = iB_rec(indices);

iC_rec_angleOfInterest = iC_rec(indices);

MMFA_rec = NA_rec.*iA_rec_angleOfInterest'; % Transposed

MMFB_rec = NB_rec.*iB_rec_angleOfInterest';

MMFC_rec = NC_rec.*iC_rec_angleOfInterest';

### Total MMF Represented by Fourier Series at Angles of Interest

totalMMF_rec = MMFA_rec + MMFB_rec + MMFC_rec;

figure;

tiledlayout(3, 4, 'TileSpacing', 'Compact', 'Padding', 'Compact');

sgtitle('Total MMF at Different ωt Degrees');

titleHandle.FontSize = 13;

for k = 1:length(angleOfInterest)

nexttile;

plot(wt_degrees, totalMMF_rec(k,:), 'LineWidth', 1.2);

hold on

plot(wt_degrees, totalMMF(k,:), 'LineWidth', 0.8);

hold off

ylim([-2 2]);

yticks(-2:0.5:2)

grid on;

xlabel("Spatial Angle (°)");

ylabel("MMF (A·Turns)");

title(sprintf('ωt = %.0f°', angleOfInterest(k)))

end

### Table

Space Harmonics (on the top horizontal axis): These refer to the harmonic components of the winding functions. The harmonics listed here include 1st, 2nd, 3rd...all the way to 13th order harmonics.

Time Harmonics (on the left vertical axis): These correspond to the harmonic components of the current in the winding. The harmonics listed here include 1st, 2nd, 3rd...all the way to 13th order harmonics.

harmonic_matrix = zeros(orderOfHarmonics(end));

diagnal_harmonics = diag(NA_bn.*iA_bn + NB_bn.*iB_bn + NC_an.*iC_bn);

cross_harmonics = NA_bn.*iA_bn' + NB_bn.*iB_bn' + NC_an.*iC_bn';

harmonic_matrix(harmonic_matrix==0) = cross_harmonics(harmonic_matrix==0)

harmonic_matrix = 13×13

-0.7526   -0.0005    0.1829    0.0004    0.0404   -0.0003   -0.1073 ⋯

-0.0000   -0.0000    0.0000   -0.0000    0.0000   -0.0000   -0.0000

0.0000   -0.0000    0.0000    0.0000   -0.0000    0.0000    0.0000

0.0000   -0.0000    0.0000    0.0000   -0.0000    0.0000    0.0000

-0.0000   -0.0000    0.0000    0.0000   -0.0000    0.0000   -0.0000

0.0000    0.0000   -0.0000   -0.0000   -0.0000    0.0000    0.0000

-0.0000   -0.0000    0.0000    0.0000   -0.0000   -0.0000   -0.0000

-0.0000   -0.0000    0.0000    0.0000    0.0000   -0.0000   -0.0000

0.0000    0.0000   -0.0000   -0.0000    0.0000   -0.0000    0.0000

0.0000    0.0000   -0.0000   -0.0000   -0.0000    0.0000    0.0000

⋮

% I'm confident with this method

function [a0,an,bn] = computeHarmonicsAmplitude(expr,wt,orderOfHarmonics)

a0 = mean(expr);

% Initialization

an = zeros(size(orderOfHarmonics));

bn = zeros(size(orderOfHarmonics));

for n = 1:length(orderOfHarmonics)

% Amplitude of even harmonics (cos terms)

an(n) = 2*mean(expr.*cos(n*wt));

% Amplitude of odd harmonics (sin terms)

bn(n) = 2*mean(expr.*sin(n*wt));

end

end